Cisco Router Lab 1 and 2 NAT Configuration

The first step in any NAT configuration is to define the inside and outside interfaces. It is imperative that we define the these interfaces for NAT overload to function.

Set the Gigabit ethernet 0/0 interface as the inside interface:

Configure terminal

Interface  G0/0

Ip nat inside

Then set the serial interface S0/0 as the outside interface:

Interface s0/0/0

Ip nat outside

Then to map server 5 to the public IP address of s0/0/0 so that it can be accessed as a web server from outside:

ip nat inside source static tcp 192.168.10.2   80  212.5.8.2 80 extendable

Now need to create an Access Control List (ACL) that will include the private network. This ACL will be applied to the NAT service and used to control the hosts that will be able to access the outside network:

Access-list 100  permit  ip  192.168.10.0    0.0.0.255  any

Now enable NAT overload and bind it to the outside interface:

Ip  nat  inside  source  list  100  interface  serial0/0/0  overload

To verify that NAT is working:

Show ip nat translations