Lab: Windows DHCP Server for Acme

Introduction

Acme will need a DHCP server at each of its sites so that workstations can receive a dynamic IP address automatically (rather than you having to go around configuring each IP manually).

(Note though the servers will need static IP addresses, or a reserved dynamic IP).

This lab will enable you to learn how to configure the first DHCP server.

Objectives

- To investigate the configuration of a DHCP server for one of Acme’s sites

- To understand the principles of DHCP servers

- To configure and troubleshoot a DHCP server to issue IP addresses dynamically to clients

Note: the primary objectives are to understand the concepts, not necessarily to get things working perfectly – so please do not panic if you cannot get everything to work!

Requirements of the ACME DHCP Server

- The DHCP server has been allocated an IP address of 199.200.X.3 (where X is a number unique to your group at this site – see your tutor if in doubt about the number).

- The DHCP server must be configured to provide a scope (range) of dynamic IP addresses for up to 250 client computers on the network.

- The addresses 199.200.X.10 and 199.200.X.20 must be excluded from this scope because they are already allocated to file servers on the network (and therefore these addresses must not be issued to DHCP clients)

- The DHCP server must inform the clients of the address of the router and the DNS server on the network (in the form of scope options):

  - The address of the router on the network is 199.200.X.1

  - The address of the DNS server for the network is 199.200.X.2

- The DHCP server must also inform clients of the ACME DNS domain name “acme.com”.

- The workstation that you connect to the network will have a dynamic address, but you must ensure that you always reserve the address 199.200.X.25 for it within the scope (based on its MAC address)

What you must do:

- You must find out how to configure a DHCP server to meet the above requirements.

- (With your group or working with a partner) allocate one virtual machine running Windows 2019 Server as your DHCP server.

- You will need to add DHCP as a server role on your server first (leave the configuration blank when you do this) – once the DHCP role is added set up the server from Administrative Tools.

- Add Scope for the DHCP and provide range of IP addresses.

- Allocate another PC running another virtual machine (2019, Windows 7 or 10) as a workstation that you will use to test the correct operation of your DHCP server.

- Connect the two computers using a crossover cable (see your tutor).

- You must configure a DHCP server according to these requirements and test it by ensuring that the workstation has received the correct address.

- On the Client configure the network TCP/IP as Obtain IP address automatically.

- use IPCONFIG/ALL on the workstation which should show the IP address of provide scope/ Range on DHCP Server.

- When you have done this demonstrate your work to your tutor

- Only when you have demonstrated your server please stop the DHCP server and remove the scope (otherwise your PC may not work properly next time you use it!). Then disconnect the cross over cable and reconnect both PCs back to the network before you leave the lab.

Take screenshots, write up an explanation of the lab in your personal log, and try to record video wherever necessary. All this will feed to your training package material.